3218 JH

DC axial compact fan

Ebm-papst

Nominal data

Type

3218 JH

VDC

VDC

min-1

W

°C

°C

m3/h

B

dB(A)

48

36 .. 53

6800

9.5

-20

70

146

6.4

55

Nominal voltage

Nominal voltage range

Speed

Power input

Min. ambient temperature

Max. ambient temperature

Air flow

Sound power level

Sound pressure level

ml = max. load · me = max. efficiency · rfa = running at free air · cs = customer specs · cu = customer unit

Subject to alterations

Web data sheet D · Page 1 of 4

ebm-paps

3218 JH

DC axial compact fan

Technical features

Dimensions

General description

92 x 92 x 38 mm

Particular design features:

Rigid compression curve for high air flow at high counterpressure.

Innovative impeller with winglets for low noise.

Optional Vario-Pro: Highly flexible software configuration for the fan ensures an easily customisable

solution to meet the individual requirements of your application.

General features:

Material: fibreglass-reinforced plastic. Impeller PA, housing PBT.

Electronic commutation completely integrated.

Protected against reverse polarity and locking.

Connection via single strands AWG 24 (H3 and H4: AWG 22), TR 64, bared and tin-plated.

Air exhaust over bars. Rotational direction clockwise looking at rotor.

Mass: 240 g (H3 and H4: 280 g).

Single strands AWG 24, TR 64, bared and tin-plated.

Right, looking at rotor

Air exhaust over bars

Ball bearings

70000 h

35000 h

0.240 kg

Fiberglass-reinforced PBT plastic

Fiberglass-reinforced PA plastic

Protected against reverse polarity and locking.

VDE, CSA, UL

Connection line

Direction of rotation

Direction of air flow

Bearing

Lifetime L10 at 40 °C

Lifetime L10 at maximum

temperature

Mass

Housing material

Material of impeller

Motor protection

Approval

Web data sheet D · Page 2 of 4

ebm-papst

3218 JH

DC axial compact fan

Product drawing

Web data sheet D · Page 3 of 4

ebm-papst

3218 JH

DC axial compact fan

Charts: Air flow

(in H ² O)

0,1

0,3

0,5

0,7

200

160

120

80

40

pf S

Pa

0

15

30

45

60

75

CFM

m³/h

VS

•

20

40

60

80

100

120

Web data sheet D · Page 4 of 4

ebm-papst S